# Materials list

Prices listed in RMB unless stated otherwise. This is because most of the materials were sourced from Taobao. Prices were accessed 11/5/2025. Total cost of US$1192.53 is obtained using the exchange rate 1 CNY=0.1491 USD as of October 3, 2026 (CNY-USD X-RATE, 2026). This cost only includes the materials listed in Table 1 and Table 2. The costs of the 3D printer, fluorescent lamp and laser goggles are not included as they are auxiliary equipment for fabrication, calibration and safety. The costs of the optical platforms and the base are not listed as they are not essential to the design and may be replaced by alternative mounting arrangements like an optical table.

Table 1. Materials used and their specifications.

| Item | Model | Specifications | Brand | Price of 1 piece (RMB) | No. pieces needed | Total price (RMB)* |
| --- | --- | --- | --- | --- | --- | --- |
| Laser | 细光款 | 532nm, 200mW DPSS laser module | 高品质激光器激光模组专门店, Taobao | 245 | 1 | 245 |
| Mirrors | GMH11-025A-AG Φ25.4mm，厚度4mm | 25.4mm diameter; thickness 4mm | 恒洋光学旗舰店, Taobao | 155 | 2 | 310 |
| Dichroic beam splitter | HGMP-550-D25，Φ25.0mm，中心波长550nm | 25mm diameter; long-pass beam splitter; central wavelength 550 nm | 恒洋光学旗舰店, Taobao | 231 | 1 | 231 |
| 45 degrees holder | HKCB1-1 SM1压圈式 厚度≤4mm | 25.4mm diameter; 45 degrees holder; 30mm cage system | 恒洋光学旗舰店, Taobao | 771.42 | 2 | 1,542.84 |
| Imaging lens | KBX058 | Bi-Convex Lens; 25.4 mm diameter; 75.6 mm EFL; Uncoated | Newport | USD $43 | 1 | USD $43 |
| Lens holders | HCP24 顶丝式镜架 直径25.4 | 25.4mm diameter; 30mm cage system | 恒洋光学旗舰店, Taobao | 113 | 3 | 339 |
| Diffraction grating | HGR01-2525-1250 | Dimensions 25x25x6mm, 1200 lines per mm, blaze wavelength 500 nm | 恒洋光学旗舰店, Taobao | 1220 | 1 | 1220 |
| Diffraction grating holder | MTMQ-1S 夹持25*25mm加锁紧 | Diffraction grating holder for 25x25mm. | 恒洋光学旗舰店, Taobao | 528 | 1 | 528 |
| Collimation lens | KPX088 | Plano-Convex Lens, N-BK7, 25.4 mm Diameter, 75.6 mm EFL, Uncoated | Newport | USD $36 | 1 | USD $36 |
| Re-imaging lens | KPX094 | Plano-Convex Lens, N-BK7, 25.4 mm Diameter, 100 mm EFL, Uncoated | Newport | USD $36 | 1 | USD $36 |
| Edge filter | M25*0.5滤光镜 | 532nm edge filter | 一佳摄影器材城, Taobao | 130 | 1 | 130 |
| Edge filter holder | MLNR-1 夹持25.4mm | Holder for 25.4 mm optics | 恒阳光学旗舰店, Taobao | 69.23 | 1 | 69.23 |
| Optical slit | ST01OH20 内径20微米 | 20 mm diameter, 20 μm wide slit | 北京世强精机科技有限公司, Taobao | 165 | 1 | 165 |
| Optical slit conversion holder | ST02PS 25.4针孔座 | Holder for 20mm diameter slit | 北京世强精机科技有限公司, Taobao | 42.5 | 1 | 42.5 |
| Optical pole | HCT-300-P4 φ6，L300mm，4根装 | 300mm optical pole | 恒阳光学旗舰店, Taobao | 372 | 1 | 372 |
| Camera | 黑白 JHEM504GM | Monochrome camera; Sony IMX264 sensor | 深圳京航工业相机直销店, Taobao | 1480 | 1 | 1480 |
| Camera lens | 50mm 1/2" C口 | 50mm focal length; 1/2” diameter; C type adapter | 安耐科技, Taobao | 65 | 1 | 65 |

Table 2. Materials required for cuvette assembly.

| Item | Model | Specifications | Brand | Price of 1 piece (RMB) | No. pieces needed | Total price (RMB) |
| --- | --- | --- | --- | --- | --- | --- |
| Lens holders | HCP24 顶丝式镜架 直径25.4 | 25.4mm diameter; 30mm cage system | 恒洋光学旗舰店, Taobao | 113 | 2 | 226 |
| Retaining ring | SM05LRR 卡环 φ12.7mm | 1/2” inner diameter | 恒阳光学, Taobao | 19.78 | 1 | 19.78 |
| Retaining ring adapter | SM1A6 SM1外螺纹/SM05内螺纹 | SM1 outer/SM05 inner | 恒阳光学, Taobao | 133.52 | 1 | 133.52 |
| Cage assembly rods | HCT-050-P4 φ6，L50mm，4根装 | 50mm length; 6mm diameter | 恒阳光学旗舰店, Taobao | 108 | 1 | 108 |

Table 3. Additional equipment required.

| Item | Model | Specifications | Brand | Price of 1 piece (RMB) | No. pieces needed | Total price (RMB) |
| --- | --- | --- | --- | --- | --- | --- |
| Optical fibre | VIS-NIR(NA 0.37)SMA905-FC/PC, 0.5m | 0.5m SMA905-FC/PC; for 600μm | 深圳市鑫锐光子科技有限公司, Taobao | 235 | 1 | 235 |
| Adapter for optical fibre | PMSMA SMA接口，25.4外径 | 25.4 outer diameter; FC/PC & SMA connection; 30mm | 恒阳光学旗舰店, Taobao | 170 | 1 | 170 |
| Pinholes | ST01OH20 内径20微米 | 20μm hole diameter, 25.4mm outer diameter | 北京世强精机科技有限公司, Taobao | 165 | 2 | 330 |
| Holder for pinholes | ST02PS 25.4针孔座 | 25.4mm diameter | 北京世强精机科技有限公司, Taobao | 42.5 | 2 | 85 |
| Screws | M4x14[50个] | M4 14mm | 固万基官方旗舰店, Taobao | 5.6 | 1 | 5.6 |
| Screws | M6*8[50个] | M6 8mm | 固万基官方旗舰店, Taobao | 5 | 1 | 5 |
| Screws | M3x20[50个] | M3 20mm | 固万基官方旗舰店, Taobao | 5.8 | 1 | 5.8 |
| Screws | M4x10[50个] | M4 10mm | 固万基官方旗舰店, Taobao | 4.9 | 1 | 4.9 |

References

- CNY-USD X-RATE. (2026, October 3). Bloomberg. Retrieved October 3, 2026, from https://www.bloomberg.com/quote/CNYUSD:CUR